# _Testing Checklist - Portola v1.1

Last updated: 9-8-2022 by Gary Withey

For all communication steps specified below, use the Slack channel:  # testing-alpine_portola

# Before Arrival

- Confirm Heaters turned on (text Randy & Theresa if confirmation not received by 8am)

## Upon Arrival										Time:

- Review freezer sensor data to ensure no thawing occurred

- Prepare fresh 10% Bleach Solution, preferably daily, but at least every 3 days

- Clean Inactivation Table thoroughly - first with 10% bleach, then with 70% ethanol or IPA

- Set up Inactivation Table

- Check sufficient 1XISS (or make) and Rxn Mix (frozen or source tubes), and not expired

- Verify sufficient test consumables for usage (1 mL tips, 10 uL tips, gloves, wipes, aluminum foil)

- Verify that you have expected tubes from staff

## At Carillon / Ormondale

- Travel to each class and set up at drop off point					Time:

- Intake Collection tubes already dropped off

- Note any anomalies (e.g., uncollected tubes)

- Continue intaking as participants drop off tubes until cut-off time 9:05am

- Communicate any intake anomalies (QR code and anomaly description)

## Upon Return to Portola									Time:

- Turn off air purifiers before beginning the test procedure.  Keep the door closed

- Double check water bath / heater  is up to temperature (at least 95°C) before starting to process samples

- Run test

- When on amp, communicate time expected to remove from amp

- Check with FL Program Admin if positives are known or unknown. Rerun if unknown

- Communicate Result image & lookup map				Time:

  - Mark result on lookup map as a circled ‘P’ or ‘I’ at the end of the tube code

  - Wait 5 mins for confirmation of whether any positives are known positives

- Communicate re-run plan for all inconclusives and unknown positives

I.e.: “re-running 1 positive, 1 inconclusive in triplicate”

- Re-run test

- Communicate re-run results ETA

- Communicate final results and time					Time:

- Return cold blocks to freezer

- Return pipettes, tip boxes, racks

- Check that all thawed inactivation solution bottles & vials are covered in foil

- Discard sample tubes from refrigerator (neg. in trash, pos. in bag in bottom freezer)

- Turn off heaters

- Empty and clean tip buckets with bleach and ethanol

# Weekly Checklist - PV - v1.1

Last updated: 9-8-2022 by Gary Withey

- Change water in water bath weekly (at Alpine)

- Preferable to use distilled water

- Update complete Inventory, communicate that Inventory tab has been updated

- Audit freezer sensor data for anomalies

- Check Expiration dates of reagents

- Empty trash bins

- OPTIONALLY: Prepare reaction tubes for the week, and freeze

- AS NEEDED:  receive deliveries

  - Use included ship list to verify contents

  - Communicate any discrepancies

  - Update Inventory with new materials

# Site Readiness Checklist - Portola - v1.1

Last updated: 9-8-2022 by Gary Withey

The following should be in place at all times to ensure smooth operation of the lab:

- Glove boxes BOTH in assay prep area AND amplification area (keep separate glove supplies)

- Wipes present (one active, plus extra on supply shelf)

- Empty Tip box filled with strip tubes (fill tip box + half source box)

- Falcon Tubes Filled with strip tube caps (two full falcon tubes, + half source box)

- 1.5mL tubes closed and in cleaned, labeled cryobox (1 active box, Next box fully ready)

- At least 5 blank Amp run forms, and at least 1 spare of both table forms (1X ISS & RM)

- Pens and markers clean and in proper place

  - Pens tested and functional

- Spare tube of inactivation solution in freezer

- Freezer thermometer tested and functional

- Lightbox battery charged and functional (if applicable)

- Old non-significant samples clear from fridge

- Tip buckets empty